FT 256 
FLIGHT INSTRUCTOR AIRPLANE
King Povenmire

http://www.lanecc.edu/flight/links.htm

Lesson 1

Practical Test Standards (PTS)

Behavioral Standards For Each Maneuver

INTRODUCTION

Special Emphasis Items

- Positive Aircraft Control

- Positive Exchange of Controls

- Stall/Spin Awareness

- Collision Avoidance

- Wake Turbulence Avoidance

- LAHSO

- Runway Incursion Avoidance

- CFIT

- Aeronautical Decision Making ADM

- Checklist Usage.

- Other areas deemed appropriate

Flight Instructor Responsibility

Certify competency

Examiner Responsibility

Test each area at the correlation level of learning

Flight Instructor Practical Test Standard

Includes all maneuvers from the Private and Commercial PTS PLUS

Technical Subject Areas

- Aeromedical Factors

- Visual Scanning and Collision Avoidance

- Principles of Flight

- Airplane Flight Controls

- Airplane Weight and Balance

- Naviation and Flight Planning

- Night Operations

- High Altitude Operations

- Federal Aviation Regulations and Publications

- National Airspace System

- Navigation Aids and Radar Services

- Logbook Entries and Certificate Endorsements

AERODYNAMICS

Flight Instructor Art

Terms

Axes of the Aircraft

Airfoil Terms

Four Forces

Stability, Maneuverability & Controllability

Aerodynamic Efficiency

Factors correlated with L/D Max

Max Range

Best glide

Minimum Total Drag

Induced Drag and Parasite Drag are equal

Specific angle-of-attack for given airfoil.

L/D Max Speed is Related to weight

L/D ratio is independent of weight

Computing L/D

Cessna 152 at 60 kts 1.5nm per 1000 ft

9:1

What happens with a 10 kt headwind?

7.5:1

Glide Speeds – Vy vs. Minimum Sink Speed

L/D = Horizontal Ft per Min / Vert Spd

At 60 kts = 6080 / 675 = 9.1   (10 kt HW = 5066 / 675 = 7.5:1)

At 70 kts = 7093 / 800 = 8:1   (10 kt HW = 6080 / 800 = 7.6:1)

Min Sink At 53 kts = 5371 / 650 = 8.2:1

Lift and Drag Coefficients

See Figure 19 in the supplement

Load Factors and Maneuvering Speed

√ Load Factor    *   Stall Speed

Maneuvering speed = Stall Speed at 3.8 or 4.4 G

√ 4.4    =    2.097

PA28RT201 Vs1 = 58 KIAS

2.097 * 58 KIAS =   121.66 KIAS

The same formula works for any stall related speed.

Short Field Approach Speed is 74 at 2750 lb

What is it at 2200 lb?

√ (2200/2750)   *  74 KIAS

√ .80   *   74 KIAS

.894    *   74 KIAS

66.18 KIAS

(Handbook value = 67 KIAS)

Does this work for Takeoff Speeds?

Well, sorta

Takeoff is related to AOA

And Power/Thrust Available

(Check the Arrow speeds – it is pretty close)

Stall Speed During a Turn

What would be your stall speed if an F-16 which normally stalls at 130 kts while in a 9-G turn?

3  *  130  =  390 kts.

How about an Arrow in a 45   bank?

√ 1.41  *  58 KIAS

1.18   *   58   =   69 KIAS

Climb Performance

Vx or Vy?  Refer to Figure 27

Vx is determined by max excess thrust

Vy is determined by max excess power

What’s the difference?

1 – HP = 33,000lb raised 1 ft/min

Thrust is a force which changes the velocity of a mass.

Measured in lbs, but has no element of time or rate.

Ground Effect

At less than 1/2 wing span Spanwise flow is changed.

Reduces both upwash and downwash

Reduces effective angle of attack

Reduces induced drag

Aerodynamic Pressures – How to determine the “Center of Pressure”

Add up all the pressures from each element

Lift and Drag

How does a wing generate lift

Where is the “equal and opposite reaction?”

Why don’t the molecules really meet?

A new look at Wind Tunnels – What if the air stands still?

How does a wing generate lift?

Where is the “equal and opposite reaction?”

Why don’t the molecules really meet?

Balance of Forces

When are Lift and Gravity equal?

When are Thrust and Drag equal?

What happens to straight and level flight when we reduce power (Pitch remains constant)?

What happens when we increase pitch only?

What happens when we increase power only?

Static Stability

Negative Static

Neutral Static

Positive Static

Dynamic Stability

Negative Dynamic

Neutral Dynamic

Positive Dynamic

V-G Diagram
(Figure 17 pg 11)

Stall speed at 1-G

Stall speed at 3.8-G

Stall speed at 0-G

How is Va determined?

How is Vno determined?

High Lift Devices

Flaps (fig 23)

Slots

Slats

Flight Instructor Art

Airfoil

Airplane

Turning vectors

Show a diagram explaining the equal and opposite reaction to Lift?