AIRCRAFT PERFORMANCE

Outline

- Density Altitude

- Takeoff

- Climb

- Range

- Glide

- Stall

- Landing

- Multi-Engine

- Wind Components

Density Altitude

- Factors

- Determining

- Effect

Factors That Affect Density Altitude

- Pressure

- Temperature

- Humidity

How much does humidity affect Density Altitude?

Determining Density Altitude

- Use chart on Figure 24

- Use Flight Computer

Effect of Higher Density Altitude on Performance

- Accelleration rate will be slower

- Engine and propeller efficiency is reduced

- Indicated Airspeed will be the same.

- Air that affects the wing is the same density as that affecting the pitot tube

Takeoff Performance

- Solve using Figure 26

- Solve using Figure 28

- Be aware of all working factors

Takeoff Performance

Page 4-8    6754    (Refer to figure 26)

Temp:		25C

PA:           	2000

GW           	2200

Headwind 	15 Kts

Sea Level 15C    2500 ft 10C

GW    Wind     Grnd   Total      Grnd   Total

2200      15        205      460        245     525

Note: Increace distance 10% for each 14° C above standard

Takeoff Performance

Page 4-10    6762    (Refer to figure 28)

Temp:       	25C	Asphalt

PA:           	2500

GW           	5500

Headwind 	2Kts Tailwind

20°C    	    30°C

GW    P.Alt     Grnd   Total      Grnd   Total

5500    2000     1680    2150      1810   2300

3000     1860    2380      2000   2550

Check ALL the notes!

How would you handle a real world situation?

McKenzie Bridge:  2600 ft long, Dry Grass

The up-hill wind is blowing your hair around a little

GW	5423

PA	1650

Temp	27°C

Climb Performance

- Vx or Vy?  Refer to Figure 27

Helicopter Performance

- What happens to Vne as you climb?

- What Vy in the Robinson?

- What in Vx in the Robinson?

- How do you take off in high density altitude?

Glide Distance

- See Figure 29

- 10% per 10 kts. makes sense at 122 kts.

Stall Speed *  Square Root of the Load Factor

- Refer to Figure 25

Landing Performance

- See Figure 31.  (How about in the real world?)

Multi-Engine Performance

- Single-Engine Service Ceiling  + 50 fpm

- Red Line = Vmc

- Blue Line = Vyse

- when you lose one engine -

50% loss of power

80% or more loss of performance

Factors Affecting Vmc

- Max rated power (sea level, standard day)

- Critical engine windmilling

- Max Gross Weight - Most Adverse Weight

- Most rearward CG

- Flaps at Takeoff setting

- Landing Gear retracted

- No more than 5°  bank toward the good engine

- For certification – No more than 20° Hdg Change!

Wind Components

- Rules of Thumb

Degrees Off Nose	Percentage XWind

- 50

45			70

60			80

- Refer to Figure 30

- 10% of your TAS off your wing = 6° of drift

- Try WIND CORRECTION/GROUNDSPEED on your calculator

TC = 000	TAS= 100

WD=090	WV =   10  Try 20, 30, 40 and 50.

GS = ?

(If more than 50% you’re not going anywhere)

Weight and Balance

All you need to know is Weight Times Arm Equals Moment

(W x A = M) “WAM”

Effects of Exceeding Balance Limits

- Too Far Aft

- Light control pressures

- Stall/Spin recovery problems

- Loss of stability

- Too far Forward

- Unable to flare completely for landing

Effects of Aft Loading (within limits)

- Reduced Stability

- Lower Stall Speed

- Stall/Spin recovery more difficult

- Higher Cruise Speed

- Lighter Elevator Control Forces

- Greater Elevator Control Effectiveness

CG Axis

- The CG is computed along the LONGITUDINAL axis

- And for Helicopters, along the LATERAL axis as well

Effect of Landing Gear Movement in the PA28RT201?

Student Misconceptions

- What difference does it make where the datum is?

- Why isn’t it always at the fulcrum?

- Why not put it at the CG?

- Why don’t we just add up the Arms & divide by the Gross Weight?

Simple Demonstration of Datum Movement effects

Fulcrum = Datum

100  lb * -10”   = -1000  lb/in

10  lb * 100”  =   1000  lb/in

110  lb              =         0  lb/in

0  / 110  =  0”

Move Datum to the Left

100  lb *  0”		=       0

10  lb * 110”		= 1100

110 lb           		= 1100 lb/in

1100 lb/in   / 110  lb	= 10 in

Move Datum to the Right

100  lb * -110” 	 =  -11000  lb/in

10  lb *      0” 	 =           0  lb/in

110  lb                  	 =  -11000  lb/in

-11000 lb/in   / 110 lb =  -100  in

MOVING or ADDING WEIGHT

Known Weight

Weight to be moved from “A” to “B”

Take the weight out of “A” and add it to “B”

Wt	Arm		Moment

GW	   1000	11.2		11200

“A”	-      12	100		 -1200

“B”	+     12	  10		    120

New CG 1000			10120

Shifting Weight

Weight		Arm		Moment

7500 lbs	80.5 in		603750 in/lbs

What do you want the moment to be?

7500 lbs	79.5		596250 in/lbs

Find the Difference		    7500 in/lbs

How far will you move the stuff? (150-30  = 120)

7500 in/lbs   /  120 in  =  62.5 lbs

N366LC

WT	x	ARM	=	MOMENT

BEW		1822	 	  89	 	162167

PILOTS	  170	 	  80.5	 	  13685

PASS	  	  400		118		  47240

BAGS	  	  100		142	  	  14280

ZFW		2492		  95	 	237372

FUEL	  	 300		  95	  	  28500

RAMP	 	2792	 	  95	 	265872

Max Gross	2750  Aft Lim	  93

How many gallons  do you have to defuel?

Did that bring us within CG limits?

How many pound of baggage do you have to move to the front seat?

Weight Added or Removed

Add or subtract both the weight and the moment

MAC = TEMAC-LEMAC

- MAC = Mean Aerodynamic Chord

- TEMAC = Trailing Edge MAC

- LEMAC = Leading Edge MAC

Mean Aerodynamic Chord
MAC = TEMAC – LEMAC