AIRCRAFT SYSTEMS

Outline

- Magnetic Compass

- Pitot-Static

- Gyroscopes

- Electrical System

- Oxygen Systems

- Power Plant

- Propellers

- Critical Engine of a Multi-Engine

Magnetic Compass

- Variation

- Deviation

- Dip

- Northerly Turning Error

- Acceleration Error

- Oscillation

- Confusation

Variation

- Easy correction:  Use Isogonic Lines

- T-V-M-D-C: Add West

- C-D-M-V-T: Add East

Deviation

- Compass Correction Card

- “For” - “Steer”

- How do you set your Gyro HDG Indicator?

Dip

- Point as exactly as you can to the Magnetic North Pole.

- Universally mounted compasses follow lines of magnetic Flux.

- BNAS: Roll out Before North, After South

- ANDS:

- Accelerate = North

- Decelerate = South

Oscillation

- Find the “edges” of the oscillations

- Average them out.

Pitot - Static System

- Airspeed

- Altimeter

- Vertical Velocity Indicator

Flight Instructor Art

- Draw the Pitot Static System Schematically

Airspeed Indicator

- Va is not shown on the Airspeed Indicator.

- Red Radial Line?

- Vne

- Vmc in Muli-Engine

- Blue Radial Line?

- Vyse in Multi-Engine

Airspeed Indicator

- Subject to errors at High Angle of Attack

- If Pitot Tube is sealed, ram air pressure remains constant.

- What happens in a climb with sealed Pitot?

- Airspeed increases due to reduced static pressure.

Altimeter

- What is standard Sea Level Pressure?

- How do 14.7 psi and 29.92 “ Hg relate?

- What about 1013.2?

- Why does it change?

- Where is  1/2 atmosphere?

- How do you explain Density Altitude?

- Why High to low and Hot to cold - - ?

Altimeter

- High to Low Lookout Below

Altimeter

- Hot to Cold, Look Out Below

Gyroscopic Instruments

- Sources:

- Vacuum

- Electricity

- Speed of Rotation Increases Both

- Precession

- Rigidity in Space

- Only one instrument uses Precession as a tool

Electric Gyro

- Turn Needle/Turn Coordinator

- Operates on precession.

- Rotation rate is critical

- Spinning too fast, standard rate indication?

- = slower rate of turn.

- Spinning too slow, standard rate indication?

- = faster rate of turn.

Turn Needle vs. Turn Coordinator

Vacuum Gyros

- Attitude Indicator

- Heading Indicator

- Rely on rigidity in space

- We can Set the GHI

- How does the GAI know where level is?

Attitude Gyro

- Air is forced onto rotor then goes through center axis and out slits on four sides

- Pendulous Vanes

- If Axis Not Vertical

- 90° around rotation closes up, other side opens

- Precession

Draw the Vacuum System

- Should operate at 4.75 to 5.25 hg

- What happens if it is outside these limits?

- Low = more wobbling

- High = more wear.

Electrical System

- Memorize these three facts

- A Generator doesn’t start to generate until reaching approximately 1100 rpm.  An alternator produces electrical power at low rpm.

- The engine can run with the battery and alternator turned off.

- (Generators were replaced with alternators after 1964.)  Watch out for old airplanes)

Oxygen Systems

- “Aviators Breathing Oxygen”

- Continuous Flow Below FL240

- Diluter systems are usually installed in jets

- Pressure Demand systems installed in jets which can operate above FL450

Oxygen Checklist

P	Pressure

R	Regulator

I	Indicators

C	Controls

E	Emergency

Engine Systems

- Four Strokes

- Ignition

- Fuel System

- Fuel-Air Mixture Control

- Induction System

- Detonation

- Electrical System

Four Stroke Internal Combustion Engine

Intake		Exhaust

- Intake			OPEN	  	CLOSED

- Compression			CLOSED	CLOSED

- Power			CLOSED	CLOSED

- Spark ignites fuel

- Exhaust			CLOSED	OPEN

Ignition System

- A proper mixture burns blue

- Excessively Rich Mixture Causes Plug Fouling

- Dual Ignition Provides Better Ignition

- Ground Wire Required to Shut Off Magneto

Mag Switch

Fuel Induction System

- Fuel injection sprays fuel into cylinders Eliminates induction system icing

- Provides better distribution

- Carburetor - float or pressure type?

- Float, works just like the toilet bowl

- Pressure, works just like an institutional toilet

Mixture

- Ratio WEIGHT of fuel : WEIGHT of air

- A given volume of air has fewer molecules at altitude.  (It weighs less per cubic inch)

- Lean to Peak EGT then enrich a specified number of degrees

Manifold Pressure

- What does it read when stopped

- Turbocharger driven by exhaust gasses

- Turbocharger will keep MP constant until waste gate is fully open (on some models)

- Turbocharger can exceed engine limits

Is Manifold Pressure Gage Required in the PA28RT201?

DETONATION

- Explosion of fuel rather than even burning

- Orange flame rather than blue

- Causes extreme pressures in cylinders

- Causes power reduction

- It “Pings.”  You can hear it in your car, but not in an airplane.

Electricity

- What’s the difference between a Generator and an Alternator

- What’s the difference between a Starter and Alternator?

- What happens when you keep the starter engaged after the engine starts?

Excessive Engine Temperatures

- Caused by Mixture Too Lean

- Or by Too Much Power

- Or by Too Slow an Airspeed

Will Cause:

- Increased Fuel Consumption

- Increased Oil Consumption

- Possible Permanent Damage

Propellers

- Design

- Forces

- Constant Speed Propellers

- Critical Engine of Multi-Engine Airplane

Propeller Design Terms

- Efficiency = Thrust HP / Brake HP

- Blade angle = chord to plane of rotation

- Effective pitch = distance actually moved

- Propeller slip = geometric pitch - effective pitch

- Why is the propeller twisted?

- To provide near constant angle of attack at cruise

Left Turning Tendencies Associated with Power (LTTAP)

- Gyroscopic Precession - When is it a LTTAP?

- Spiral Slipstream

- “P” Factor

- Torque

How could we design out some of these?

Critical Engine

- See diagram on 3-26

Constant Speed Propellers

- Changes blade angle to maintain RPM with varying airspeed and MP

- How?

Max power is absorbed at small angle of attack, high RPM

Over Square

- Is a good rule of thumb for an airplane that doesn’t have a power chart.

- Good rule of thumb for A.R.S. pilots (Airplane Repossession Specialists).

- Over Square is more efficient if allowed by the manufacturer.

- 65% in the Arrow is ±26”mp and 2200 rpm