# Flight Instruments

## Magnetic Compass

## Pitot-Static

## Gyroscopes

# Magnetic Compass

## Variation

## Deviation

## Dip

### Northerly Turning Error  -  Acceleration Error  - Oscillation

# Variation

## Easy correction:  Use Isogonic Lines

## TVMDC: Add West

## Timid Virgins Make Dull Company At Weddings

## CDMVT: Add East

## Can Dead Men Vote Twice At Elections

# Deviation

## Compass Correction Card

## “For” - “Steer”

## How do you set your Gyro HDG Indicator?

# Dip

## Point as exactly as you can to the Magnetic North Pole.

## Universally mounted compasses follow lines of magnetic Flux.

## BNAS: Roll out Before North, After South

## ANDS:

### Accelerate = North

### Decelerate = South

# Oscillation

## Find the “edges” of the oscillations and average them out.

# Pitot - Static System

## Airspeed

## Altimeter

## Vertical Velocity Indicator

# Flight Instructor Art

## Draw the Pitot Static System Schematically

# Altimeter

## What is standard Sea Level Pressure?

## How do 14.7 psi and 29.92 “ Hg relate?

## Why does it change?

## Where is  1/2 atmosphere?

## How do you explain Density Altitude?

## Why High to low and Hot to cold - - ?

## High to Low Lookout Below

## Hot to Cold, Look Out Below

# Gyroscopic Instruments

## Sources:

### Vacuum

### Electricity

## Speed of Rotation Increases Both

### Presession

### Rigidity in Space

## Only one instrument uses Presession as a tool

# Electric Gyro

## Turn Needle/Turn Coordinator

## Operates on presession.

## Rotation rate is critical

## Spinning too fast, standard rate indication?

## = slower rate of turn.

## Spinning too slow, standard rate indication?

## = faster rate of turn.

# Vacuum Gyros

## Attitude Indicator

## Heading Indicator

## Rely on rigidity in space

## We can Set the GHI

## How does the GAI know where level is?

# Attitude Gyro

## Air is forced onto rotor then through center axis and out slits on four sides

# Pendulous Vanes

## 90° around rotation closes up, other side opens

## Presession

# Flight Instructor Art

# Draw the Vacuum System

## Vacuum System

## Should operate at 4.75 to 5.25 hg

## What happens if it is outside these limits?

## Low = more wobbling

## High = more wear.