NAVIGATION

Outline

- Enroute Flight  - Sectionals, “TVMDCAW”

- Wind Triangle - (Flight Instructor Art)

- Flight Computer

- Off-Course Correction

- Radio Navigation

- Area Navigation (RNAV)

Sectional Chart

Please get out your Klamath Falls Sectional

WIND TRIANGLE
TC=032  FD=1927  TAS=104

- Draw Course Line

- Plot Wind Direction and Velocity from Starting point.

- Place a plotter with at end of wind arrow (C)

- Rotate until TAS intersects course (D)

- Draw TH line (C to D)

- Measure GS from A to D

- Measure Wind Correction Angle from     TC to TH

Flight Computer

- Bring Extra Batteries, or

- Learn to use the Whiz-Wheel

- Use your computer to solve each type problem in the Test Prep

Try Some Computer Exercises

FD 1325  TAS 130  TC 100°  TH = ?  GS = ?

A      23nm       B                 93 nm                C

Time from A to B is 10 min.  GS = ?

Time from B to C will be ?

FD 2409-40  FL190  IAS = 190.  TAS = ?

Density Altitude = ?

60 : 1

Off Course Correction  60 to 1

- Off-course Correction = Parallel + Converge

A to B is 23 NM.  B to C is 93 NM  Time from A to B is 10 min.  GS = ?

Time from B to C will be ?

ADF

C = D

When the Correction = the Deviation from your nose or tail, your are on course.

- Picture the location of the station out the window

- “If I turned my airplane until the ADF centered

- What would my heading be?”

- For the test, verify by adding or subtracting Relative Bearing from Magnetic Heading.

Time To Station = (Minutes * 60) / Change

- Time = Seconds / Change

- (T=S/C)

Correct for known winds

ADF

- Relative Bearing.  Mentally move the needle to the Heading Indicator

VOR

- Explain “Reverse Sensing”

- HSI eliminates Reverse sensing on ILS ...

- If the OBS is set to the FRONT COURSE

RMI

- RMI points to the VOR just like an ADF

- What happens when the compass fails?

- VOR needle points to the NUMBER

- ADF needle points to the DIRECTION

For Example

- Compass not slaved, and in error

DME

- UHF Channel paired with VHF Omni Range

- Coded ID transmitted once for each three or four VOR ID’s

- Line of Sight accuracy is better than 1/2 mile or 3% of the distance whichever is greater

DME

- Slant Range

- Greatest at high altitude directly over VORTAC

- At 12,000 AGL  You will never get closer than 2 nm, and your groundspeed will slow to a stop as you cross.

Area Navigation (RNAV)

- The needle deflection indicates Nautical Miles Off Course rather than degrees

- Nautical - (RNAV grew up after the conversion to Knots.)

- GPS Standardization nightmare

- CFI must be able to “learn” the student’s system, and then teach it.