PROCEDURES AND AIRPORT OPERATIONS

Outline

- Airspace

- A/FD

- NOTAMS

- Communications

- Airport Lighting and Marking

- Segmented Circle

- Wake Turbulence

- Flight Plans

Airspace

- ABCDEG

- A = Above All – IFR Only

- B = Bodacious – Clearance Required, Mode-C 
         3-miles & Clear of Clouds

- C = Communication – “3 – 152” Mode-C

- D = Dialogue  “3 – 152”

- E = Everywhere Else IFR & VFR separated by rule

- G = Go if you dare “1 mile CC”

- Terminal Radar Service Area (TRSA) Sorta C

Purpose of Controlled Airspace

- Separation of IFR and VFR traffic

- Scenarios

SVFR

Flying under Class - B

IFR in Class – G

Special Use Airspace

- Prohibited Area

- Restricted Area

- Warning Area

- National Security Area

- Military Operations Area (MOA)

- Alert Area

- Controlled Firing Area

- Wilderness Area

Other Airspace

- Airport Advisory Area/Information

- Military Training Routes

Four-Number –Below 1500 agl

THREE - Number Surface to 10,000 ft

Other Airspace

- Temporary Flight Restrictions

- Parachute Jump Aircraft Operations

- Published VFR Routes

- Flyways

- Corridors

- Transition Routes

A/FD

- Pilot  Controlled Lighting

- Runway Gradient

- Control Tower Operating Hours

Pilot Controlled Lighting

- Seven Times for Highest

- Three Times for Lowest

- Can you turn down the lights?

- How  long do they stay on?

Runway Gradient

- Furnished if greater than 0.3%   (15 ft per NM)

Tower Operating Hours

- Listed in ZULU time

- Not  to be confused with “D/E surface” Effective Hrs

NOTAM

- NOTAM (L)

- NOTAM (D)

- FDC NOTAM

- Published NOTAM

- TFR

What Type NOTAM?

- TXWY CHARLIE CLOSED

- ILS GS OTS 1400-2000Z

- RWY 34 CLOSED

- AIRPORT UNLIGHTED SS TO SR

FLIGHT PLANS

- VFR - Held for 1 hr.

- They will be looking for you after --- ?

- IFR - The RNAV codes have changed

- You will not be given question 7133

- Under what conditions must you CLOSE an IFR flight  plan?

GROUND OPERATIONS Runway Incursion Prevention

All Airports

- Preflight review of airport layouts

- Review NOTAM for runway/taxiway closures

- Beacon or Strobe ON while taxiing

- Understand airport signage.

Tower Controlled Airports

- Read back all hold short instructions.

- Read back all runway crossing instructions.

- Clear active runway AQAP, then . . .

- Wait for taxi instructions before next intersection.

- Write down complex taxi instructions.

- Ask for Progressive Taxi if unsure.

RIGHT OF WAY RULES
Aircraft on the ground give way to landing aircraft.

- Two aircraft on the ground - Avoid each other.

- (UAL and AAL)

ARE YOU SURE YOU KNOW WHERE YOU ARE?

ARE YOU SURE THAT ATC KNOWS WHERE YOU ARE?

LAX

“TAXI  INTO POSITION AND HOLD.”

“BIGJET 1243 CLEARED TO LAND.”